RISC-V VeeR EL2 Based Real-Time Data Buffering SoC

1. INTRODUCTION

1.1 Project Overview

The proposed project is a RISC-V VeeR EL2 Based Real-Time Data Buffering System-on-Chip (SoC) designed to acquire, temporarily store, transfer, process, and report continuous real-time data from an external source. The architecture is intended for a Real-Time Industrial Motor Vibration Data Acquisition, Buffering, and Monitoring application in which a 3-axis accelerometer is associated with an industrial motor and continuously produces vibration samples.

The main challenge in this application is that vibration data is generated continuously, while the processor may simultaneously be executing application instructions, performing control operations, or processing previously acquired data. If the processor is required to handle every incoming sample individually, a significant amount of processor time is consumed by repetitive data-movement operations. This can reduce processor availability for application processing and may also result in inefficient data handling when the incoming data rate is higher than the rate at which the processor can respond.

To overcome this problem, the proposed SoC separates data movement, processor control, and event handling. Dedicated hardware is used for buffering incoming samples, transferring buffered data to memory, and notifying the processor about important hardware events. This allows the processor to concentrate on system control and application-level processing while dedicated hardware handles continuous data movement.

The proposed SoC uses the VeeR EL2 RISC-V processor as the central processing unit. The processor executes the application software, configures the hardware IPs, accesses memory and peripheral registers, handles interrupts, processes the acquired vibration data, and reports system information. The VeeR EL2 is a 32-bit RV32IMC processor with a four-stage scalar in-order pipeline.

The complete SoC contains the mandatory components required for the project: VeeR EL2 RISC-V Processor, Instruction Memory, Data Memory, AXI4 Bus Interconnect (2 Masters, 8 Slaves), UART, Timer, and GPIO. In addition, three application-specific IP blocks are integrated into the architecture: FIFO, DMA Controller, and Interrupt Controller.

The three additional IPs are not included only to satisfy the requirement of integrating additional IP blocks. Each IP directly solves a specific problem associated with continuous real-time data handling:

FIFO is used for temporary storage of incoming data.

DMA Controller is used for hardware-assisted transfer of buffered data to Data Memory.

Interrupt Controller is used to notify the processor when important hardware events require processor attention.

These three IPs together form the Real-Time Data Buffering Subsystem of the SoC.

1.2 Application of the SoC

The selected application is Real-Time Industrial Motor Vibration Data Acquisition, Buffering, and Monitoring.

In this application, a 3-axis accelerometer is associated with an industrial motor. The accelerometer continuously measures vibration produced by the motor along its three axes and provides digital measurement samples.

The accelerometer is selected because it provides a realistic source of continuous real-time sensor data, which is well suited for demonstrating the buffering and data-transfer capabilities of the proposed SoC. The purpose of this project is not to design the internal electronics or communication protocol of the physical accelerometer. Instead, the accelerometer is treated as the external real-time data source that supplies digital samples to the SoC.

For RTL implementation and verification, the physical accelerometer is represented by a testbench-generated data source. The testbench generates vibration-sample values and applies them to the same digital input interface that represents the external sensor connection.

The current SoC input interface uses:

data_in[31:0] — 32-bit incoming sample data.

valid_in — indicates that the input data contains a valid sample.

Therefore, the physical sensor interface is abstracted from the SoC data-buffering architecture. The current project begins at the point where a valid digital sample is available at the SoC input.

The application-level flow is:

Industrial Motor → 3-Axis Accelerometer → Digital Vibration Samples → SoC Input Interface → FIFO → DMA Controller → Data Memory → VeeR EL2 Processor → Monitoring/Processing → UART → External Host/PC.

1.3 Problem Statement

A real-time vibration sensor continuously generates measurement samples. These samples must be captured in the correct order and retained until the system is ready to transfer and process them.

A processor-only implementation would require the VeeR EL2 processor to repeatedly monitor the incoming data, receive each sample, store the sample, transfer the sample to memory, and repeat the same operation for every incoming data value.

This creates unnecessary processor workload because the processor spends execution cycles performing repetitive data-movement operations instead of application processing.

Another problem occurs when the external data source continues to produce samples while the processor or memory-transfer mechanism is temporarily busy. If the incoming data cannot be accepted or transferred at the required rate, samples may be lost or the system may become inefficient.

Therefore, the proposed SoC requires a hardware-assisted mechanism that can:

Receive continuous data → Temporarily buffer the data → Transfer data efficiently → Notify the processor when required → Allow the processor to process the acquired data.

The architecture addresses this requirement by introducing a FIFO for buffering, a DMA Controller for data transfer, and an Interrupt Controller for event notification.

1.4 Proposed Solution

The proposed solution divides the responsibilities of the system between the VeeR EL2 processor and dedicated hardware IP blocks.

The VeeR EL2 processor is responsible for:

System initialization

Hardware configuration

Application execution

Interrupt handling

Accessing transferred data

Application-level data processing

Status and result reporting

The dedicated hardware IPs are responsible for:

Temporary buffering of incoming data

Automatic movement of buffered data to memory

Detection and notification of important hardware events

This creates a clear separation between control operations and continuous data movement.

The three additional IPs operate together as follows:

FIFO → Temporarily stores incoming vibration samples

DMA Controller → Transfers buffered samples to Data Memory

Interrupt Controller → Notifies VeeR EL2 about important hardware events

The resulting architecture reduces the need for the processor to perform repetitive data-transfer operations.

1.5 Purpose of the FIFO

FIFO stands for First-In First-Out.

The FIFO is the first hardware block in the SoC data path after the external digital sample interface.

The primary purpose of the FIFO is to provide temporary storage for continuously arriving sensor data.

When a valid vibration sample arrives at the SoC, the sample is written into the FIFO. The FIFO stores the samples in the same order in which they are received. Therefore, the oldest stored sample is the first sample that is removed from the FIFO for further transfer.

The FIFO is required because the external data source and the internal data-transfer mechanism may not always consume and produce data at exactly the same rate. A new sample may arrive while the DMA Controller is temporarily busy or while a previous transfer is still being completed. Instead of losing the incoming sample immediately, the FIFO provides temporary storage.

The FIFO maintains status information that allows the system to determine the current condition of the buffer. Important FIFO conditions include:

Empty — no valid sample is currently stored.

Full — all available FIFO storage locations are occupied.

Data Count — indicates the number of currently stored entries.

Threshold Condition — indicates that the FIFO has accumulated a programmed amount of data.

These conditions are used to manage data availability and buffer overflow situations.

The purpose of the FIFO can therefore be summarized as:

Continuous Sensor Input → Temporary Storage → Controlled Data Availability.

1.6 Purpose of the DMA Controller

DMA stands for Direct Memory Access.

The DMA Controller is responsible for transferring buffered data from the FIFO data path to Data Memory without requiring the VeeR EL2 processor to manually transfer every individual data word.

Without a DMA Controller, the processor would need to repeatedly execute operations similar to:

Read Sample → Write Sample to Memory → Read Next Sample → Write Next Sample → Repeat.

This increases processor workload and consumes processor execution cycles.

With the DMA Controller, the processor configures the required transfer information before the transfer starts. The configuration includes:

Source Address

Destination Address

Transfer Length

Control Information

After the DMA transfer is started, the DMA Controller performs the programmed data movement.

The processor is therefore not required to execute a software instruction for every individual sample transfer.

The primary data-transfer path becomes:

FIFO → DMA Controller → Data Memory.

The DMA Controller therefore acts as the hardware data-transfer engine of the real-time buffering subsystem.

1.7 Purpose of the Interrupt Controller

The Interrupt Controller is responsible for managing hardware events that require attention from the VeeR EL2 processor.

Without interrupt-based operation, the processor could continuously poll hardware status registers to determine whether an event has occurred.

For example, the processor would repeatedly need to ask:

Is the FIFO ready?

Has the FIFO reached its threshold?

Has the DMA transfer completed?

Did a DMA transfer error occur?

Continuous polling consumes processor execution time.

The Interrupt Controller provides an event-driven mechanism instead. Hardware blocks generate interrupt requests when configured events occur. The Interrupt Controller collects, manages, and forwards the appropriate interrupt request to the VeeR EL2 processor.

Important interrupt-generating events may include:

FIFO threshold condition

FIFO full condition

DMA transfer completion

DMA transfer error

Other configured hardware events

The Interrupt Controller provides interrupt management functionality including interrupt enable, pending, and clear operations.

The basic interrupt path is:

Hardware Event → Interrupt Controller → VeeR EL2 Processor.

This allows the processor to respond to important events without continuously polling every hardware block.

1.8 Why the Three Additional IPs Are Used Together

The three additional IPs solve three different problems within the same real-time data-handling system.

FIFO — Buffering

The FIFO solves the problem of temporary data storage. It temporarily stores incoming samples when the next stage is not immediately ready to consume them.

DMA Controller — Data Transfer

The DMA Controller solves the problem of efficient data movement. It transfers buffered samples to Data Memory without requiring the processor to manually move every sample.

Interrupt Controller — Event Notification

The Interrupt Controller solves the problem of processor notification. It informs the processor when an important event requires software attention.

Therefore, the three IPs work together as:

External Data Source → FIFO → DMA Controller → Data Memory

while the event path is:

FIFO/DMA Events → Interrupt Controller → VeeR EL2

and the processor control path is:

VeeR EL2 → System Bus → FIFO/DMA/Interrupt Controller.

The three IPs therefore form the central application-specific hardware subsystem of the SoC.

1.9 Role of the VeeR EL2 RISC-V Processor

The VeeR EL2 processor is the central processing and control element of the SoC.

The processor does not continuously transfer every incoming sensor sample. Instead, it configures and controls the dedicated hardware and performs the application-level processing.

The VeeR EL2 performs the following major functions:

1. Initializes the SoC and its peripherals.

2. Configures the FIFO operation.

3. Configures the DMA source, destination, and transfer length.

4. Configures and enables the required interrupts.

5. Responds to FIFO and DMA interrupt events.

6. Reads status information from the hardware IPs.

7. Accesses the vibration data stored in Data Memory.

8. Performs application-level monitoring or processing.

9. Reports transfer and application status through UART.

The processor communicates with the memories and peripherals through the System Bus using memory-mapped registers and memory locations.

The processor therefore acts as the system controller and application processor, while the FIFO and DMA Controller handle the repetitive real-time data movement.

1.10 Role of the Mandatory SoC Components

Instruction Memory

Instruction Memory stores the application program executed by the VeeR EL2 processor.

The processor fetches its instructions from Instruction Memory during program execution.

Data Memory

Data Memory stores application variables, processing data, and the vibration-data blocks transferred by the DMA Controller.

The VeeR EL2 processor accesses the stored vibration samples from Data Memory for application-level processing.

System Bus Interconnect

The System Bus Interconnect provides the communication path between the SoC Masters and Slaves using the AXI4 protocol. It performs address decoding and routes transactions. In this architecture, there are 2 Masters (VeeR EL2 Processor, DMA Controller) and 8 Slaves (Instruction Memory, Data Memory, UART, Timer, GPIO, FIFO Control, DMA Control, Interrupt Controller).

UART

UART provides serial communication between the SoC and an external host or PC.

It can be used to report:

Data-transfer status

DMA completion status

Error information

Processed vibration-data information

System status

Application results

Timer

The Timer provides timing functionality and periodic timing events required by the application.

It may be used for periodic operations, timing measurements, or other software-controlled timing functions.

GPIO

GPIO provides general-purpose digital input/output functionality.

It can be used for basic control signals, external digital inputs, and status indication such as LEDs.

1.11 Overall System Operation

The complete system operation begins with the industrial motor producing mechanical vibration.

The 3-axis accelerometer measures the vibration and produces digital measurement samples. In the RTL environment, the testbench models this external sensor and generates equivalent digital sample values.

The digital samples are presented to the SoC through the external input interface.

When valid_in indicates that data_in[31:0] contains a valid sample, the sample is accepted by the FIFO.

The FIFO temporarily stores the incoming samples in their arrival order.

When the required data is available, the DMA Controller performs the programmed transfer of samples from the FIFO data path to Data Memory.

The DMA Controller performs the repetitive data movement without requiring the VeeR EL2 processor to manually transfer every sample.

When an important event occurs, such as a DMA transfer completion, DMA transfer error, or configured FIFO condition, an interrupt request is generated.

The Interrupt Controller receives and manages the interrupt request and provides the appropriate interrupt to the VeeR EL2 processor.

The processor responds to the interrupt, reads the relevant status information, accesses the transferred vibration data stored in Data Memory, and performs the application-level monitoring or processing.

The processor can then report the transfer status and application information through UART to an external PC or monitoring terminal.

The overall application flow is:

Industrial Motor → 3-Axis Accelerometer → Digital Vibration Data → SoC Input Interface → FIFO → DMA Controller → Data Memory → VeeR EL2 Processor → Monitoring/Processing → UART → External Host.

1.12 Data, Control, and Interrupt Paths

The architecture is separated into three major paths so that data movement, processor control, and hardware event notification can be clearly distinguished.

Data Path

The data path carries the actual vibration samples:

External Data Source → FIFO → DMA Controller → Data Memory.

This path is responsible for moving sensor data from the external input to memory.

Control Path

The control path is generated by the VeeR EL2 processor through the memory-mapped System Bus:

VeeR EL2 → System Bus → FIFO / DMA Controller / Interrupt Controller.

This path is used to configure hardware registers, start operations, read status, and control the data-buffering subsystem.

Interrupt Path

The interrupt path carries hardware-event notifications toward the processor:

FIFO / DMA Events → Interrupt Controller → VeeR EL2.

This path allows the processor to respond to important events without continuously polling the hardware.

1.13 Sensor Interface Scope

The physical accelerometer and its physical communication protocol are considered outside the current SoC architecture scope.

The current SoC begins at the point where a synchronized digital sample is available through the defined external input interface.

The current external interface consists of:

data_in[31:0] — 32-bit incoming sample data.

valid_in — indicates that the input sample is valid.

clk — SoC system clock.

reset_n — SoC reset.

The project does not require an internal I2C Controller, SPI Controller, or dedicated sensor-interface IP solely to represent the accelerometer.

For RTL verification, the testbench directly models the external sensor data source and drives the defined digital input interface.

This keeps the scope of the project focused on:

Real-Time Data Acquisition + FIFO Buffering + DMA-Based Data Transfer + Interrupt-Driven Control + Processor-Based Monitoring.

1.14 Clock and External Data Consideration

The external physical sensor may internally operate using its own timing or communication mechanism. However, the current SoC architecture does not directly implement the physical sensor communication interface.

The data presented to the SoC shall therefore satisfy the defined SoC input timing requirements and shall be available in the SoC clock domain before being accepted by the FIFO.

For the RTL project, the testbench shall generate data_in[31:0] and valid_in according to the SoC clock.

Therefore, the current architecture does not require Kiro to invent a separate clock-domain-crossing mechanism for the external sensor unless a future version of the architecture explicitly introduces an asynchronous sensor interface.

1.15 Overall Project Objective

The objective of the project is to design and verify a complete RTL-based SoC using the provided VeeR EL2 RISC-V processor as the central processing unit and integrating FIFO, DMA Controller, and Interrupt Controller as the three additional application-specific IP blocks.

The completed SoC shall:

Receive continuous digital vibration samples from an external data source.

Temporarily buffer incoming samples using the FIFO.

Transfer buffered samples to Data Memory using the DMA Controller.

Notify the processor about important hardware events using the Interrupt Controller.

Allow the VeeR EL2 processor to configure and control the hardware subsystem.

Allow the processor to access and process the acquired vibration data.

Report system, transfer, and monitoring information through UART.

Provide GPIO and Timer functionality as required by the SoC architecture.

Be verified using RTL simulation and embedded software execution.

2. Block Diagram

3. Signal List

3.1 External Input Signals

| Signal Name | Direction | Width | Source | Destination | Purpose |
| --- | --- | --- | --- | --- | --- |
| clk | External -> SoC | 1 | External Clock Source | All SoC Logic | Main system clock |
| reset_n | External -> SoC | 1 | External Reset Source | All SoC Logic | Active-low system reset |
| data_in[31:0] | External -> SoC | 32 | External Data Source / Testbench | FIFO Input | Incoming real-time sample data |
| valid_in | External -> SoC | 1 | External Data Source / Testbench | FIFO Input | Indicates data_in contains a valid sample |
| gpio_in[7:0] | External -> SoC | 8 | External Digital Source | GPIO | External GPIO input signals |
| uart_rx | External -> SoC | 1 | Host / External Device | UART | UART serial receive input |

3.2 External Output Signals

| Signal Name | Direction | Width | Source | Destination | Purpose |
| --- | --- | --- | --- | --- | --- |
| data_ready | SoC -> External | 1 | FIFO/Input Logic | External Data Source | Indicates FIFO can accept a new sample |
| uart_tx | SoC -> External | 1 | UART | Host / PC | Serial transmission of status/data information |
| gpio_out[7:0] | SoC -> External | 8 | GPIO | External Device / Status LEDs | Digital output or status indication |

3.3 External Data Source to FIFO Signal Flow

| Signal Name | Direction | Width | Source | Destination | Purpose |
| --- | --- | --- | --- | --- | --- |
| data_in[31:0] | External -> FIFO | 32 | External Data Source | FIFO | Carries incoming vibration sample |
| valid_in | External -> FIFO | 1 | External Data Source | FIFO | Indicates input sample is valid |
| data_ready | FIFO -> External | 1 | FIFO / Input Logic | External Data Source | Indicates FIFO can accept a new sample |
| fifo_wr_en | Internal | 1 | Input Logic | FIFO | Requests a FIFO write when input handshake succeeds |
| fifo_wdata[31:0] | Internal | 32 | Input Logic | FIFO | Data written into FIFO |

3.4 FIFO Interface Signals

| Signal Name | Direction | Width | Source | Destination | Purpose |
| --- | --- | --- | --- | --- | --- |
| fifo_wr_en | Input to FIFO | 1 | Input Logic | FIFO | Requests write of incoming data |
| fifo_wdata[31:0] | Input to FIFO | 32 | Input Logic | FIFO | Data written into FIFO |
| fifo_rd_en | DMA -> FIFO | 1 | DMA Controller | FIFO | Requests oldest FIFO entry |
| fifo_rdata[31:0] | FIFO -> DMA | 32 | FIFO | DMA Controller | Oldest buffered data word |
| fifo_empty | FIFO -> Internal | 1 | FIFO | DMA / Interrupt / Status | Indicates no valid data is stored |
| fifo_full | FIFO -> Internal | 1 | FIFO | Input / Interrupt / Status | Indicates FIFO is full |
| fifo_count | FIFO -> Internal | Parameterized | FIFO | DMA / Control | Number of valid entries stored |
| fifo_threshold | FIFO -> Internal | 1 | FIFO | Interrupt Controller | Programmed threshold condition |
| fifo_overflow | FIFO -> Internal | 1 | FIFO | Interrupt Controller / Status | Write attempted when FIFO unavailable/full |
| fifo_underflow | FIFO -> Internal | 1 | FIFO | Interrupt Controller / Status | Read attempted when FIFO is empty |

3.5 DMA Controller Interface Signals

| Signal Name | Direction | Width | Source | Destination | Purpose |
| --- | --- | --- | --- | --- | --- |
| dma_start | Internal | 1 | DMA Control Register | DMA Controller | Starts programmed DMA operation |
| dma_enable | Internal | 1 | DMA Control Register | DMA Controller | Enables DMA operation |
| dma_src_addr | Internal | Parameterized | DMA Register | DMA Controller | Source address configuration |
| dma_dst_addr | Internal | Parameterized | DMA Register | DMA Controller | Destination address configuration |
| dma_length | Internal | Parameterized | DMA Register | DMA Controller | Number of data units to transfer |
| dma_busy | DMA -> Internal | 1 | DMA Controller | Status / Processor | Indicates DMA transfer is active |
| dma_done | DMA -> Internal | 1 | DMA Controller | Interrupt Controller / Status | Indicates transfer completion |
| dma_error | DMA -> Internal | 1 | DMA Controller | Interrupt Controller / Status | Indicates DMA transfer error |
| dma_fifo_req | DMA -> FIFO | 1 | DMA Controller | FIFO | Requests next buffered data word |
| dma_fifo_valid | FIFO -> DMA | 1 | FIFO | DMA Controller | Indicates FIFO output data is valid |
| dma_fifo_data[31:0] | FIFO -> DMA | 32 | FIFO | DMA Controller | Buffered sample transferred to DMA |

3.6 DMA to Data Memory Signal Flow

| Signal Name | Direction | Width | Source | Destination | Purpose |
| --- | --- | --- | --- | --- | --- |
| m1_axi_awaddr | DMA → Bus | ADDR_WIDTH | DMA Controller | AXI Interconnect | Write address |
| m1_axi_awlen | DMA → Bus | 8 | DMA Controller | AXI Interconnect | Burst length |
| m1_axi_awsize | DMA → Bus | 3 | DMA Controller | AXI Interconnect | Burst size |
| m1_axi_awburst | DMA → Bus | 2 | DMA Controller | AXI Interconnect | Burst type |
| m1_axi_awvalid | DMA → Bus | 1 | DMA Controller | AXI Interconnect | Write address valid |
| m1_axi_awready | Bus → DMA | 1 | AXI Interconnect | DMA Controller | Write address ready |
| m1_axi_wdata | DMA → Bus | DATA_WIDTH | DMA Controller | AXI Interconnect | Write data |
| m1_axi_wstrb | DMA → Bus | STRB_WIDTH | DMA Controller | AXI Interconnect | Write strobes |
| m1_axi_wlast | DMA → Bus | 1 | DMA Controller | AXI Interconnect | Write last transfer |
| m1_axi_wvalid | DMA → Bus | 1 | DMA Controller | AXI Interconnect | Write valid |
| m1_axi_wready | Bus → DMA | 1 | AXI Interconnect | DMA Controller | Write ready |
| m1_axi_bresp | Bus → DMA | 2 | AXI Interconnect | DMA Controller | Write response |
| m1_axi_bvalid | Bus → DMA | 1 | AXI Interconnect | DMA Controller | Write response valid |
| m1_axi_bready | DMA → Bus | 1 | DMA Controller | AXI Interconnect | Write response ready |

3.7 VeeR EL2 to System Bus Signal Flow

| Signal Name | Direction | Width | Source | Destination | Purpose |
| --- | --- | --- | --- | --- | --- |
| m0_axi_awaddr | VeeR → Bus | ADDR_WIDTH | VeeR EL2 | AXI Interconnect | Write address |
| m0_axi_awvalid | VeeR → Bus | 1 | VeeR EL2 | AXI Interconnect | Write address valid |
| m0_axi_awready | Bus → VeeR | 1 | AXI Interconnect | VeeR EL2 | Write address ready |
| m0_axi_wdata | VeeR → Bus | DATA_WIDTH | VeeR EL2 | AXI Interconnect | Write data |
| m0_axi_wstrb | VeeR → Bus | STRB_WIDTH | VeeR EL2 | AXI Interconnect | Write strobes |
| m0_axi_wvalid | VeeR → Bus | 1 | VeeR EL2 | AXI Interconnect | Write valid |
| m0_axi_wready | Bus → VeeR | 1 | AXI Interconnect | VeeR EL2 | Write ready |
| m0_axi_bresp | Bus → VeeR | 2 | AXI Interconnect | VeeR EL2 | Write response |
| m0_axi_bvalid | Bus → VeeR | 1 | AXI Interconnect | VeeR EL2 | Write response valid |
| m0_axi_bready | VeeR → Bus | 1 | VeeR EL2 | AXI Interconnect | Write response ready |
| m0_axi_araddr | VeeR → Bus | ADDR_WIDTH | VeeR EL2 | AXI Interconnect | Read address |
| m0_axi_arvalid | VeeR → Bus | 1 | VeeR EL2 | AXI Interconnect | Read address valid |
| m0_axi_arready | Bus → VeeR | 1 | AXI Interconnect | VeeR EL2 | Read address ready |
| m0_axi_rdata | Bus → VeeR | DATA_WIDTH | AXI Interconnect | VeeR EL2 | Read data |
| m0_axi_rresp | Bus → VeeR | 2 | AXI Interconnect | VeeR EL2 | Read response |
| m0_axi_rvalid | Bus → VeeR | 1 | AXI Interconnect | VeeR EL2 | Read valid |
| m0_axi_rready | VeeR → Bus | 1 | VeeR EL2 | AXI Interconnect | Read ready |

3.8 Bus to Peripheral Signal Flow

| Signal Name (Per Slave 0-7) | Direction | Width | Source | Destination | Purpose |
| --- | --- | --- | --- | --- | --- |
| s_axi_awaddr | Bus → Slave | ADDR_WIDTH | AXI Interconnect | Selected Slave | Target write address |
| s_axi_awvalid | Bus → Slave | 1 | AXI Interconnect | Selected Slave | Write address valid indicator |
| s_axi_awready | Slave → Bus | 1 | Selected Slave | AXI Interconnect | Slave ready to accept address |
| s_axi_wdata | Bus → Slave | DATA_WIDTH | AXI Interconnect | Selected Slave | Write data payload |
| s_axi_wstrb | Bus → Slave | STRB_WIDTH | AXI Interconnect | Selected Slave | Write byte strobes |
| s_axi_wvalid | Bus → Slave | 1 | AXI Interconnect | Selected Slave | Write data valid |
| s_axi_wready | Slave → Bus | 1 | Selected Slave | AXI Interconnect | Slave ready for data |
| s_axi_bresp | Slave → Bus | 2 | Selected Slave | AXI Interconnect | Write transaction status response |
| s_axi_bvalid | Slave → Bus | 1 | Selected Slave | AXI Interconnect | Response valid |
| s_axi_bready | Bus → Slave | 1 | AXI Interconnect | Selected Slave | Bus ready for response |
| s_axi_araddr | Bus → Slave | ADDR_WIDTH | AXI Interconnect | Selected Slave | Target read address |
| s_axi_arvalid | Bus → Slave | 1 | AXI Interconnect | Selected Slave | Read address valid |
| s_axi_arready | Slave → Bus | 1 | Selected Slave | AXI Interconnect | Slave ready for read address |
| s_axi_rdata | Slave → Bus | DATA_WIDTH | Selected Slave | AXI Interconnect | Read data payload |
| s_axi_rresp | Slave → Bus | 2 | Selected Slave | AXI Interconnect | Read transaction status response |
| s_axi_rvalid | Slave → Bus | 1 | Selected Slave | AXI Interconnect | Read data valid |
| s_axi_rready | Bus → Slave | 1 | AXI Interconnect | Selected Slave | Bus ready for read data |

3.9 Instruction Memory Signal Flow

| Signal Name | Direction | Width | Source | Destination | Purpose |
| --- | --- | --- | --- | --- | --- |
| imem_addr | VeeR EL2 -> IMEM | Parameterized | VeeR EL2 | Instruction Memory | Instruction fetch address |
| imem_req | VeeR EL2 -> IMEM | 1 | VeeR EL2 | Instruction Memory | Instruction fetch request |
| imem_rdata | IMEM -> VeeR EL2 | Instruction width | Instruction Memory | VeeR EL2 | Instruction returned to processor |
| imem_ready | IMEM -> VeeR EL2 | 1 | Instruction Memory | VeeR EL2 | Indicates instruction fetch completion |
| imem_error | IMEM -> VeeR EL2 | 1 | Instruction Memory | VeeR EL2 | Indicates instruction access error |

3.10 Data Memory Signal Flow

| Signal Name | Direction | Width | Source | Destination | Purpose |
| --- | --- | --- | --- | --- | --- |
| dmem_addr | Master -> DMEM | Parameterized | CPU / DMA | Data Memory | Memory address |
| dmem_wdata | Master -> DMEM | Parameterized | CPU / DMA | Data Memory | Data to be written |
| dmem_write | Master -> DMEM | 1 | CPU / DMA | Data Memory | Indicates write transaction |
| dmem_req | Master -> DMEM | 1 | CPU / DMA | Data Memory | Requests memory access |
| dmem_rdata | DMEM -> Master | Parameterized | Data Memory | CPU / DMA | Read data |
| dmem_ready | DMEM -> Master | 1 | Data Memory | CPU / DMA | Indicates access completion |
| dmem_error | DMEM -> Master | 1 | Data Memory | CPU / DMA | Indicates memory access error |

3.11 Interrupt Controller Input Signals

| Signal Name | Direction | Width | Source | Destination | Purpose |
| --- | --- | --- | --- | --- | --- |
| fifo_irq_threshold | FIFO -> IC | 1 | FIFO | Interrupt Controller | FIFO threshold interrupt |
| fifo_irq_full | FIFO -> IC | 1 | FIFO | Interrupt Controller | FIFO full interrupt |
| fifo_irq_overflow | FIFO -> IC | 1 | FIFO | Interrupt Controller | FIFO overflow interrupt |
| dma_irq_done | DMA -> IC | 1 | DMA Controller | Interrupt Controller | DMA completion interrupt |
| dma_irq_error | DMA -> IC | 1 | DMA Controller | Interrupt Controller | DMA transfer error |
| timer_irq | Timer -> IC | 1 | Timer | Interrupt Controller | Timer interrupt if implemented |
| gpio_irq | GPIO -> IC | 1 | GPIO | Interrupt Controller | GPIO interrupt if implemented |
| irq_pending | Internal | Parameterized | Interrupt Controller | Interrupt Logic | Pending interrupt state |
| irq_enable | CPU/Register -> IC | Parameterized | VeeR EL2 | Interrupt Controller | Enables selected interrupts |
| irq_clear | CPU/Register -> IC | Parameterized | VeeR EL2 | Interrupt Controller | Clears pending interrupts |

3.12 Interrupt Controller to VeeR EL2 Signal

| Signal Name | Direction | Width | Source | Destination | Purpose |
| --- | --- | --- | --- | --- | --- |
| irq | Interrupt Controller -> VeeR EL2 | 1 | Interrupt Controller | VeeR EL2 | Requests processor interrupt service |

3.13 UART Signal Flow

| Signal Name | Direction | Width | Source | Destination | Purpose |
| --- | --- | --- | --- | --- | --- |
| uart_rx | External -> UART | 1 | Host / External Device | UART | Serial receive input |
| uart_tx | UART -> External | 1 | UART | Host / PC | Serial transmit output |
| uart_tx_data | VeeR EL2 -> UART | UART-defined | VeeR EL2 | UART | Data to transmit |
| uart_rx_data | UART -> VeeR EL2 | UART-defined | UART | VeeR EL2 | Received data |
| uart_tx_valid | VeeR EL2 -> UART | 1 | VeeR EL2 | UART | Indicates transmit data is available |
| uart_tx_ready | UART -> VeeR EL2 | 1 | UART | VeeR EL2 | Indicates UART can accept transmit data |
| uart_rx_valid | UART -> VeeR EL2 | 1 | UART | VeeR EL2 | Indicates received data is available |

3.14 GPIO Signal Flow

| Signal Name | Direction | Width | Source | Destination | Purpose |
| --- | --- | --- | --- | --- | --- |
| gpio_in[7:0] | External -> GPIO | 8 | External Source | GPIO | Digital GPIO input |
| gpio_out[7:0] | GPIO -> External | 8 | GPIO | External Device / LED | Digital output/status indication |
| gpio_dir[7:0] | VeeR EL2 -> GPIO | 8 | VeeR EL2 | GPIO | GPIO direction configuration |
| gpio_out_reg[7:0] | VeeR EL2 -> GPIO | 8 | VeeR EL2 | GPIO | GPIO output data control |
| gpio_in_reg[7:0] | GPIO -> VeeR EL2 | 8 | GPIO | VeeR EL2 | Current GPIO input value |

3.15 Timer Signal Flow

| Signal Name | Direction | Width | Source | Destination | Purpose |
| --- | --- | --- | --- | --- | --- |
| timer_enable | VeeR EL2 -> Timer | 1 | VeeR EL2 | Timer | Enables Timer |
| timer_start | VeeR EL2 -> Timer | 1 | VeeR EL2 | Timer | Starts timing operation |
| timer_value | VeeR EL2 -> Timer | Parameterized | VeeR EL2 | Timer | Configured period/count value |
| timer_count | Timer -> VeeR EL2 | Parameterized | Timer | VeeR EL2 | Current Timer count/status |
| timer_expire | Timer -> IC | 1 | Timer | Interrupt Controller | Timer expiration event |
| timer_irq | Timer -> IC | 1 | Timer | Interrupt Controller | Timer interrupt request |

4. CONTROL PATH

The control path defines how the VeeR EL2 RISC-V processor initializes, configures, starts, monitors, and controls the memories and peripheral IP blocks present in the SoC using the AXI4 Bus Interconnect. The control path carries addresses, commands, configuration values, and status information. It is logically separate from the real-time data path, which carries the vibration samples.

The VeeR EL2 processor acts as the main processor-side control master. It accesses Instruction Memory, Data Memory, UART, Timer, GPIO, FIFO, DMA Controller, and Interrupt Controller through the System Bus Interconnect using the memory-mapped address space. The System Bus decodes the processor address and routes each transaction to the selected block.

Main control relationship: VeeR EL2 → System Bus Interconnect → Selected Memory or Peripheral

4.1 Control Path to Mandatory SoC Components

| Target Block | Processor Control | Status / Readback | Purpose |
| --- | --- | --- | --- |
| Instruction Memory | Instruction fetch request and address | Instruction data / response | Provides application instructions to VeeR EL2 |
| Data Memory | Read/write address, control, and write data | Read data / response | Stores application data and DMA-transferred vibration data |
| UART | Configuration and transmit-data writes | UART status / received data | Provides serial communication and result reporting |
| Timer | Enable, start, and timing configuration | Timer count / status | Provides timing and periodic timing functions |
| GPIO | Direction and output-data configuration | GPIO input / status | Provides digital input, output, and status indication |

4.2 Control Path to Additional IPs

| IP Block | Processor Control | Processor Reads | Purpose |
| --- | --- | --- | --- |
| FIFO | Enable/configuration and threshold settings | Empty, full, count, status | Controls and monitors the real-time input buffer |
| DMA Controller | Source address, destination address, transfer length, control, start | Busy, done, error, status | Configures and starts hardware-assisted data transfer |
| Interrupt Controller | Interrupt enable, configuration, and clear | Pending and status | Controls interrupt sources and reports hardware events |

4.3 Processor Control Sequence

1. The VeeR EL2 starts executing the application from Instruction Memory.

2. The processor initializes the required SoC peripherals.

3. The processor configures the FIFO and its required threshold or control settings.

4. The processor configures the DMA source address, destination address, transfer length, and control information.

5. The processor configures and enables the required interrupt sources.

6. The processor starts or enables normal data acquisition and transfer operation.

7. The hardware performs buffering and DMA transfer without requiring the processor to move every sample individually.

8. The processor waits for hardware status or an interrupt event.

9. When an interrupt occurs, the processor reads the interrupt status and identifies the event source.

10. The processor reads the relevant FIFO or DMA status and accesses transferred data in Data Memory.

11. The processor performs the required application-level monitoring or processing.

12. The processor reports required status or results through UART.

13. The processor clears the serviced interrupt and resumes normal execution.

4.4 Processor Write Control Flow

A processor write is used when the VeeR EL2 needs to configure a peripheral, issue a command, or store data into a writable memory location.

| Step | Source | Control Information | Destination | Result |
| --- | --- | --- | --- | --- |
| 1 | VeeR EL2 | Address + write indication + write data | System Bus | Processor transaction is presented to the bus |
| 2 | System Bus | Address decode | Selected slave | Exactly one addressed slave is selected |
| 3 | System Bus | Write control + write data | Selected slave | Selected register or memory location accepts the write |
| 4 | Selected slave | Transaction response | System Bus | Completion or error status is returned |
| 5 | System Bus | Response | VeeR EL2 | Processor continues after transaction completion |

4.5 Processor Read Control Flow

A processor read is used when the VeeR EL2 needs status information, configuration values, received data, or stored application data from a memory or peripheral.

| Step | Source | Control Information | Destination | Result |
| --- | --- | --- | --- | --- |
| 1 | VeeR EL2 | Address + read indication | System Bus | Processor read request is presented to the bus |
| 2 | System Bus | Address decode | Selected slave | Exactly one addressed slave is selected |
| 3 | Selected slave | Read data + response | System Bus | Requested information is returned |
| 4 | System Bus | Read data + response | VeeR EL2 | Processor receives the requested information |

4.6 FIFO Control Path

The VeeR EL2 controls the FIFO through its memory-mapped registers. Software configures the FIFO and reads FIFO status, while the actual sample write and read operations are performed by the hardware data path.

Processor control flow: VeeR EL2 → System Bus → FIFO Register Interface

Processor-controlled FIFO functions include: FIFO enable or control; FIFO threshold configuration; FIFO status reading; FIFO data-count monitoring; and FIFO error/status monitoring.

Important separation: Processor → FIFO is control/status access, while External Data Source → FIFO and FIFO → DMA are data-transfer paths.

4.7 DMA Controller Control Path

The VeeR EL2 configures the DMA Controller through memory-mapped registers. The processor provides the transfer parameters and then starts the DMA operation. After the transfer starts, the DMA hardware performs the repetitive data movement.

| Configuration | Written By | Destination | Purpose |
| --- | --- | --- | --- |
| Source Address | VeeR EL2 | DMA Controller | Defines the source of the transfer |
| Destination Address | VeeR EL2 | DMA Controller | Defines the memory destination |
| Transfer Length | VeeR EL2 | DMA Controller | Defines the number of data units to transfer |
| Control | VeeR EL2 | DMA Controller | Defines DMA operating control |
| Start / Enable | VeeR EL2 | DMA Controller | Starts or enables the programmed transfer |

DMA status is returned to the processor through the memory-mapped status interface. Important status conditions include DMA busy, DMA completion, and DMA error.

4.8 Interrupt Controller Control Path

The VeeR EL2 configures the Interrupt Controller through memory-mapped registers. The processor enables required interrupt sources, reads pending/status information when an interrupt occurs, services the event, and clears the corresponding interrupt source.

Processor configuration path: VeeR EL2 → System Bus → Interrupt Controller

Hardware event path: FIFO / DMA Events → Interrupt Controller → VeeR EL2

The hardware event path is an interrupt path rather than a processor-generated bus control transaction.

4.9 UART, Timer, and GPIO Control Paths

| Peripheral | Processor Control | Readback / Event | Purpose |
| --- | --- | --- | --- |
| UART | Configuration and transmit-data writes | UART status and received data | External serial communication and reporting |
| Timer | Enable, start, and period/count configuration | Current count/status and optional expiration event | Timing and periodic operations |
| GPIO | Direction and output-data configuration | Input values/status | Digital input/output and status indication |

4.10 Control Path Error Handling

If the VeeR EL2 accesses an invalid or unmapped address, the System Bus shall return an appropriate error response instead of selecting an unrelated valid memory or peripheral. Peripheral or memory transaction errors shall be returned to the processor through the bus response mechanism.

4.11 Control Path Rules for RTL Implementation

- The VeeR EL2 shall act as AXI Master 0 (s00_axi).

- The DMA Controller shall act as AXI Master 1 (s01_axi) for direct memory access.

- The AXI Interconnect shall decode the target addresses and map them across its 8 Slave output ports (m00_axi through m07_axi).

- Each peripheral and memory block shall implement an AXI4 or AXI4-Lite slave interface to accept transactions.

- Processor write transactions will utilize the AW, W, and B channels to configure the FIFO, DMA Controller, and Interrupt Controller registers.

- Processor read transactions will utilize the AR and R channels to pull status and data.

4.12 Control Path Summary

The control path is the processor-driven path used to initialize the SoC, configure the memories and peripherals, start the FIFO/DMA operation, manage interrupts, monitor status, and control application behavior.

Fundamental control relationship: VeeR EL2 → System Bus Interconnect → Selected Memory / Peripheral